# XCAM N329- SDK

| Version: | 1.2 |
| --- | --- |
| Date: | 22th March 2018 |
| Author: | LAU Yat Fei Jackson |

| Revision | Date | Comment |
| --- | --- | --- |
| 1.0 | 5 Dec 2017 | Original release |
| 1.1 | 1 Feb 2018 | UART Interface + Multiple Colour Palette + POIs |
| 1.2 | 22 March 2018 | Dual camera support + Adaptive colour palette |

Contents

XCAM Foundation	3

Library Structure	3

Single camera module	3

Dual camera module	4

Project Setting	5

Header	5

Software dependencies	5

Preprocessor Symbols	6

ThermalSensorAPI_N329.lib	6

Customised data structure	7

APIs	10

UART/HUART Interface	16

Setting	16

Output commands	16

Build and Test	17

Hybrid images overlaying	17

Procedures	17

Demonstration	18

Single camera module	18

Dual camera module (mobile app)	18

# XCAM Foundation

The documentation of the XCAM SDK for users.

The XCAM SDK basically contains fundamental functions which supports for MCU N329- series library to control XCAM. Dual camera module is using additional library, below is the main project structure of XCAM_N329.

## Library Structure

### Single camera module

|----N32901-3_Non-OS_KEIL

|----I2C

|----lib/w55fa93_i2c.h

|----HTPA32_0619_R1_XCam

|----example/SwLib3

|---- I2CS/DrvI2C_Thermal.c

|---- I2CS/DrvI2C.h

|---- main.c

|---- standalone.c

|---- Table_UVC.c

|---- usbd_video.c

|--- -wb_init.s

|---- KEIL

|----MI_Thermal.uvproj

|----MI_Thermal/MI_XCAM_N329_SW_I2C.bin

|----lib/videoclass_HTPA32.h

|----lib/w55fa93_UVC_YUB_Only.lib

|----N32901

|----N32901.sct

|----SYSLIB/Lib

|----w55fa93_reg.h

|----W55FA93_syslib.lib

|----wberrcode.h

|----wbio.h

|----wblib.h

|----wbtypes.h

|----ThermalAPI_inC

|----lib/videoclass_HTPA32.h

|----example/demo/KEIL/ThermalSensor.h

|----example/demo/KEIL/MI_XCAM.h

|----example/demo/KEIL/demo_Data/N32901_SingleTask/ThermalSensorAPI_N329.lib

|----UDC

|----lib/usbd.h

|----lib/w55fa93_UDC.lib

|----VPOST

|----lib/w55fa93_vpost.h

|----lib/W55FA93_VPOST.lib

### Dual camera module

|----N32901-3_Non-OS_KEIL

|----HTPA32_20180215_R1_DualCam

|----lib/videoclass_HTPA32.h

|----lib/w55fa93_UVC_YUB_Only.lib

|----example/HTPA32

|---- I2CS

|---- DrvI2C_Thermal.c

|---- DrvI2C.c

|---- DrvI2C.h

|---- VIN

|---- GCD.c

|---- Smpl_FSC_Ov7740.c

|---- Smpl_I2C.c

|---- VIN_demo.c

|---- vin_demo.h

|---- main.c

|---- standalone.c

|---- Table_UVC.c

|---- usbd_video.c

|--- -wb_init.s

|---- KEIL

|----MI_Thermal.uvproj

|----MI_Thermal/MI_XCAM_N329_SW_I2C.bin

|----gpio

|----lib/w55fa93_gpio.h

|----lib/w55fa93_gpio.lib

|----N32901

|----N32901.sct

|----SYSLIB/Lib

|----w55fa93_reg.h

|----W55FA93_syslib.lib

|----wberrcode.h

|----wbio.h

|----wblib.h

|----wbtypes.h

|----ThermalAPI_inC

|----lib/videoclass_HTPA32.h

|----example/demo/KEIL/ThermalSensor.h

|----example/demo/KEIL/MI_XCAM.h

|----example/demo/KEIL/demo_Data/N32901_SingleTask/ThermalSensorAPI_N329.lib

|----UDC

|----lib/usbd.h

|----lib/w55fa93_UDC.lib

|----VideoIn

|----lib/W55fa93_VideoIn.h

|----lib/W55FA93_VideoIn.lib

|----VPOST

|----lib/w55fa93_vpost.h

|----lib/W55FA93_VPOST.lib

SDK Version: Release V3.01 180322

## Project Setting

### Header

"ThermalSensor.h”

“w55fa93_vpost.h”

“videoclass_HTPA32.h”

### Software dependencies

..\..\..\lib

..\..\..\..\SYSLIB\Lib

..\..\..\..\I2C\lib

..\..\..\..\UDC\lib

..\..\..\..\VPOST\Lib

..\I2CS

..\..\..\..\ThermalAPI_inC\example\demo\KEIL

..\..\..\..\..\MI_Sensor

### Preprocessor Symbols

Define: MCU=329

### ThermalSensorAPI_N329.lib

#define DEADPIXELCOMPENSATE

Usage: Dead pixel(s) compensation using average masking.

Prerequisite: EEPROM should have the information of

1) Number of dead pixel and

2) Coordinates of dead pixels

*If there are any dead pixels next to each other, sensor should be rejected since we are using averaging masking

*Pre-set from ThermalSensorAPI_N329.lib

#define POI

Usage: Point of interests (POIs) recording for each frame. If not define, then FRAMEPOIS object will be empty.

*Pre-set from ThermalSensorAPI_N329.lib

#define COLOR_ADAPTIVE

Usage: The FRAMEPOIS determine how the temperature spread on colour palette

Better to use monotonic RGB colour table under adaptive mode, i.e.: COLORPALETTE_BW_ADAPTIVE

#define COLORPALETTE0/1/2/3/_BW_STEP10/_BW_STEP25

Usage: Define colour palette mode

### Customised data structure

peri_interface : Peripheral interface

enum peri_interface{ HUART = 0, UART, SPI };

YUV_COLOR_INFO_T: YUV data structure

typedef struct YUV_COLOR_INFO_T { unsigned int YUVData; } YUV_COLOR_INFO_T;

RGB_COLOR_INFO_T: RGB data structure

typedef struct RGB_COLOR_INFO_T{ unsigned char R; unsigned char G; unsigned char B; } RGB_COLOR_INFO_T;

REGISTERSETTING : Data structure storing the values of sensor register.

typedef struct REGISTERSETTING{ unsigned short MBIT; unsigned short BIAS; unsigned short CLOCK; unsigned short BPA; unsigned short PU; } REGISTERSETTING;

EEPROM: Data structure storing values read from EEPROM.

typedef struct EEPROM{ float PixCMin; // Minimum sensitivity coefficient, used for scaling float PixCMax; // Maximum sensitivity coefficient, used for scaling unsigned short gradScale; // Emissivity factor unsigned short TableNumberSensor; // The look-up table number of sensor belongs unsigned short epsilon; // Factor for fine tuning of the sensitivity for all Pixel REGISTERSETTING calibRegister; // Sensor register values for calibration unsigned short VddRef; // used supply voltage during calibration measured float PTATGrad; // Factor of calculating ambient temperature (Ta) float PTATOff; // Factor of calculating ambient temperature (Ta) unsigned char VddScaling; // VddComp scaling coefficient unsigned short VddScalingOff; // VddComp scaling coefficient unsigned char GlobalOffset; // Factor for fine tuning of the sensitivity for all Pixel unsigned short GlobalGain; // Factor for fine tuning of the sensitivity for all Pixel REGISTERSETTING userRegister; // Sensor register values for user unsigned short DevID; // Device ID unsigned char NrOfDefPix; // Number of dead pixel(s) unsigned short DeadPixAdr[MAXNROFDEFECTS]; // Array of dead pixel addr unsigned char DeadPixMask[MAXNROFDEFECTS]; // Array of dead pixel mask signed short VddGrad[ELAMOUNT]; // VddComp gradient signed short VddOff[ELAMOUNT]; // VddComp offset signed short ThGradN[Pixel]; // thermal gradient signed short ThOffN[Pixel]; // compensate for any thermal drifts unsigned long PixCN[Pixel]; // Sensitivity coefficients } EEPROM;

SENSORSETTING: Data structure storing sensor look-up table values for TO calculation.

typedef struct SENSORSETTING{ signed long TABLENUMBER; signed long long PCSCALEVAL; // defined scaling coefficient signed long NROFTAELEMENTS; signed long NROFADELEMENTS; signed long TAEQUIDISTANCE; signed long ADEQUIDISTANCE; signed long ADEXPBITS; signed long TABLEOFFSET; unsigned char MBITTRIMDefault; signed long SensRv; unsigned int* TempTable; unsigned int* XTATemps; unsigned int* YADValues; } SENSORSETTING;

TEMPIXEL: POI information

typedef struct TEMPIXEL{ unsigned short x; unsigned short y; signed short Tmp; } TEMPIXEL;

FRAMEPOI: Storing POIs of single frame

typedef struct FRAMEPOI{ TEMPIXEL maxTemPixel; TEMPIXEL maxTemPixel; } FRAMEPOIS;

## APIs

ThermalSensor.h

void			N329_InitSensor(void);

void			N329_OpenSensor(void);

int			N329_Interface_init (unsigned short interface);

void			N329_UartDataValid_Handler (UINT8* buf, UINT32 u32Len);

MI_XCAM.h

void 			InitI2C(unsigned char mode);

void 	HighDensSequentialRead(unsigned short address,unsigned char* data, unsigned short numbytes);

void 	HighDensPageWrite(unsigned short address,unsigned char* data, unsigned short numbytes);

void 			ReadCalibDataN(void);

void			InitSensorDev(unsigned short TN);

void 			InitMBITTRIMN(unsigned char user);

void 			InitBIASTRIMN(unsigned char user);

void 			InitBPATRIMN(unsigned char user);

void 			InitPUTRIMN(unsigned char user);

void 			InitCLKTRIMN(unsigned char user);

void 			GetImageData (void);

unsigned short	GetTemp(unsigned int x, unsigned int y);

FRAMEPOIS 		GetFramePOIs (void);

unsigned short	GetTempDisplay(void);

int			GetTargetPixelIndex(void);

unsigned int 	CalcTO(unsigned int TAmb, signed int dig, signed long PiC, unsigned int dontCalcTA);

unsigned int		StartStreaming(int Mode, char Temps, char Stream);

void			SetTempDisplay(unsigned short flag);

void			SetTargetPixelIndex(int index);

void 			ResetFramePOIs(void);

| void | N329_InitSensor(void); |
| --- | --- |

Open and setup the system environment for I2C.

| void | N329_Open(void); |
| --- | --- |

Initialization of sensor, mainly read calibration data and setup trim registers.

| int | N329_Interface_init (unsigned short input); |
| --- | --- |

Initialization of peripheral interface, input HUART/UART as parameter.

| void | N329_UartDataValid_Handler (void); |
| --- | --- |

Handle the UART/HUART command list.

| void | InitI2C(unsigned char mode); |
| --- | --- |

Setup the EEPROM & Sensor address to the device.

Dependencies:	0 Init I²C for Sensor (> 1000 kHz)

1 Init I²C for EEPROM (max 400 kHz)

| void | HighDensSequentialRead(unsigned short address,unsigned char* data, unsigned short numbytes); |
| --- | --- |
| void | HighDensPageWrite(unsigned short address,unsigned char *data, unsigned short numbytes); |

HighDensSequentialRead() & HighDensPageWrite() reads/writes multiple bytes from a high-density (>= 32 Kb) serial EEPROM device.

Dependencies: 	'address' contains address word

'data' contains the reading result

'numbytes' contains the length of bytes to read

| void | ReadCalibDataN(void); |
| --- | --- |

Read all required calibration data from sensor EEPROM, details in TYPEDEF structure EEPROM.

| void | InitSensorDev (unsigned short TN); |
| --- | --- |

Open and setup the sensor device, pick up the setting corresponding to the type of optics by TableNumber.

| void | InitMBITTRIMN(unsigned char user); |
| --- | --- |

Dependencies:	0 	Setting during Calibration

1	Setting from user

Range: 		4 <= m <= 12

Initialization of Trim Register 1 MBIT/ (m+4) bit as ADC resolution to sensor register.

| void | InitBIASTRIMN(unsigned char user); |
| --- | --- |

Dependencies:	0 	Setting during Calibration

1	Setting from user

Range:		0 to 31  1μA to 13μA

Initialization of Trim Register 2, adjust the bias current of the ADC.

| void | InitBPATRIMN(unsigned char user); |
| --- | --- |

Dependencies:	0 	Setting during Calibration

1	Setting from user

Range:		0 to 31  0.2μA to 4.0μA

Initialization of Trim Register 5, adjust the common mode voltage of the preamplifier.

| void | InitPUTRIMN(unsigned char user); |
| --- | --- |

Dependencies:	0 	Setting during Calibration

1	Setting om user

Range:		“1000” = 100 kOhm; “0100” = 50 kOhm; “0010” = 10 kOhm; “0001” = 1 kOhm

Initialization of Trim Register 7, select internal pull up resistor on SDA/SCL.

| void | InitCLKTRIMN(unsigned char user); |
| --- | --- |

Dependencies:	0 	Setting during Calibration

1	Setting from user

Range:		0 to 63  1MHz to 13MHz

Initialization of Trim Register 4, clock frequency setting CLK_TRM.

| unsigned short | GetTemp(unsigned int x, unsigned int y); |
| --- | --- |

GetTemp(x,y) gets the calculated temperature from 2D-array sensor, in single pixel.

* P.S: If you want to get the whole 2D-array, you may use

ptempBuff = (signed short*) &Target;

pixTemp = *(ptempBuff + x * (sizeof(Target[0])/sizeof(signed short)) + y) - 2732;

Dependencies: unsigned int x= x-coordinate of target pixel 	(0 < x < COLUMN-1)

unsigned int y = y-coordinate of target pixel	(0 < y < ROW-1)

Return Code:	Caelcius °C x 10 (e.g: 301 = 30.1°C, right shifted 1 decimal)

998		Coordinate input ERROR

| FRAMEPOIS | GetFramePOIs(void); |
| --- | --- |

GetFramePOIs return the FRAMEPOIS object.

Return: 	       	data structure FRAMEPOIS

| unsigned short | GetTempDisplay(void); |
| --- | --- |

GetTempDisplay() get the temperature display flag. The flag is set by SetTempDisplay().

Dependency:		/

Return: 	       	The flag of showing bracket and temperature on firmware.

| unsigned short | GetTargetPixelIndex(void); |
| --- | --- |

GetTargetPixelIndex() set which pixel temperature should be shown.

Dependency:		/

Return: 	       	The temperature value of targeted pixel.

| unsigned int | CalcTO(unsigned int TAmb, signed int dig, signed long PiC, unsigned int dontCalcTA); |
| --- | --- |

CalcTO() calculate the object temperature via look-up table.

Dependencies:  	TAmb 	= ambient temperature

dig 	= pixel voltage

PiC 	= pixel sensitivity coefficients

Return: 	       	Object Temperature in dK

| unsigned int | StartStreaming(int Mode, char Temps, char Stream); |
| --- | --- |

StartStreaming() starts the streaming of the sensor, main sequence can be seen in the interrupt.

Dependencies:   	Mode = Initialize(1) or Normal streaming (0)

Temps: whether it is calculating temperature

Stream 	= whether it is streaming mode

Return:		0x0 / 0xFF

| void | SetTempDisplay(unsigned short flag); |
| --- | --- |

SetTempDisplay() set to turn on/off of showing temperature.

Dependencies:   	flag = On(1) or Off(0)

| void | SetTargetPixelIndex(int index); |
| --- | --- |

SetTargetPixelIndex() set which pixel we want to show its temperature on the display.

Dependencies:   	0 <= index <= WIDTH*HEIGHT

-1(Default)	Average temperature of bracket area

Return value:		/

| void | ResetFramePOIs(void); |
| --- | --- |

ResetFramePOIs() reset the FRAMEPOIS object.

## UART/HUART Interface

### Setting

Main.c: main()  N329_Interface_init(UART/HUART);

Baud rate: 115200

Data: 8bit		Parity: none

Stop: 1bit		Flow control: none

UART				HUART

### Output commands

VCMD=$COMMAND

| TMP (Display Temperature) | DAT (Temperature Array) | RES (Resolution) |
| --- | --- | --- |
| SET (Sensor setting) | STA (Sensor Status) | UVC (UVC status) |

# Build and Test

- Build main project successfully

- Download binary file(.bin) to MCU

- Run the application that support UVC (e.g.: AMCAP)

*Dual camera project will only show CIS image through UVC

# Hybrid images overlaying

For dual camera module, the data streaming is different from single module. There are two different data to be handled: Thermal image and CIS image.

The thermal image data is padded into the LSB of UV data for CIS sensor in one data stream, normal UVC software will only show the CIS image streaming.

Detail is shown below:

Each temperature value is sent as 11bits/pixel, first bit represents positive/negative and the following 10 bits for temperature value.

### Procedures

- Get YUV data streaming from CIS camera on application side

- Extract thermal image data from YUV data stream.

- Convert the temperature values into RGB pixels.

- Interpolates the RGB pixels so that can be fitted into desired size, i.e.: same as CIS image (640*480) for overlaying.

- Please aware of the frame rate difference from CIS and XCAM. CIS image is in higher framerate than XCAM, i.e.: 15fps:6-8 fps

# Demonstration

### Single camera module

### Dual camera module (mobile app)